# Remote Controlled Plane and Controller

# ENGR 2999, Douglas College

# Maximus Larouche, Brendan Morrison

# 20/04/2026

# Abstract

Contents

Abstract	2

Introduction	2

Bill of Materials	2

Design	2

Mechanical Systems	2

Electrical Systems	2

Conclusion	2

# Introduction

For our project, the moto for our plane is as follows: “it only has to fly, but that doesn’t mean it has to land”. The plane was originally based around the Cessna 182 from which we steered our own design to fit our needs and constraints. The plane’s wings, aerodynamics, and power systems were originally designed for around a 1:1 thrust to weight ratio, however the final weight was around 300g less than our original target, thus resulting in a 1.5:1 ratio, putting it comfortably in the sport flyer category. The plane itself

# Bill of Materials

| Item | Quantity | Unit Cost | Cost |
| --- | --- | --- | --- |
| Soldering / Prototyping Materials | 1 (pack) | $16.99 | $16.99 |
| 1450Kv Motor | 1 | $30.81 | $30.81 |
| 60A ESC | 1 | $44.79 | $44.79 |
| 14.8v 2200mAh Battery | 1 | $32.99 | $32.99 |
| IMU/Gyroscope module (MPU6050) | 1 | $4.33 | $4.33 |
| Slide Potentiometers | 1 | $2.55 | $2.55 |
| Toggle Switches | 1 | $1.07 | $1.07 |
| NRF24L01 PA + LNA Antenna Module | 2 | $4.95 | $9.90 |
| WaveShare RP2350-Zero modules | 2 | $11.50 | $22.99 |
| Joystick Modules | 1 | $5.75 | $5.75 |
| MG90s 9g Servos | 3 | ~$4.43 | ~$13.30 |
| 9x6 Two Blade Propeller | 1 | $8.90 | $8.90 |
| Miscellaneous electrical components (already had / found them) | - The Insides of a CAT6 Ethernet Cable (wire) - SSD1306 128x84 OLED Display (1x) - EC11 Rotary Encoder (1x) - XL6009 Boost Converter (1x) - TP4056 Battery Charger (1x) - Scavenged Lithium Ion Cell (1x) - 10k Resistors (5x) - 100uF Electrolytic Capacitor (2x) - 10nF Ceramic Capacitors (7x) |  |  |
| RC Plane Landing Gear Kit | 1 |  |  |
| Total Cost: $194.37 |  |  |  |

# Design and Changelog

## Mechanical Systems

## Electrical Systems

The design and behaviour of the controller and plane circuits are essentially the same from the first and last versions (not including the prototypes in between). The communications reliability was the biggest upgrade between the two versions with it being optimized for longer ranges. For the sake of time, features from the first version were left unprogrammed (multicore, display, gyroscope). If a part is mentioned (IMU, display) then assume it is the same part from the previous version if not specified.

### Controller

| V1 | The first version was designed to only test the functionality of the antenna between the controller and a mock plane. The controller would transmit the joystick and throttle positions and receive the IMU data from the plane with the antenna. The controller would then draw the positions of the analog controls, the IMU data, and the connection status with the plane on the OLED display. The components in the circuit consisted of the following: - WaveShare RP2350-Zero (1x) - NRF24L01 PA + LNA Antenna Module (1x) - Joystick Module (1x) - Slide Potentiometer (1x) - SSD1306 128x64 OLED Display (1x) - 10nF Ceramic Capacitors (3x) |
| --- | --- |
| V2 (Final) | The second version kept all the original components with the new additions being the battery power + charging system and a rotary encoder that was never used in the program. The added components are as follows: - TP4056 Single Cell Battery Charger (1x) - XL6009 Boost Converter (1x) - Lithium Cell (unknown capacity) (1x) - Toggle switch (1x) - EC11 Rotary Encoder (1x) - 100uF Electrolytic Capacitor (1x) - Additional ceramic caps and resistors While this version had all these components, the display and rotary encoders were never given functions due to program issues. The controller will send the PWM signals or the motor and servos, along with an RTT timer for determining the input latency. The controller now also utilizes the microcontroller’s onboard LED for displaying the connection status. |

Final Controller Behaviour Description:

On startup the controller will turn on the onboard LED

### Plane

| V1 (Mock Plane) | The first version of the plane, the “mock plane”, was meant to only act as a plane by receiving the analog control data and transmitting the IMU data. It would draw a mirror of the controller’s display to show that it was properly receiving the data. The circuit consisted of the following: - Waveshare RP2350-Zero (1x) - NRF24L01 PA + LNA Antenna Module (1x) - SSD1306 128x64 OLED Display (1x) - MPU6050 Inertial Measurement Unit (1x) This was built to test the functionality of the antenna. |
| --- | --- |
| V2 (Final) | The second version was fully rebuilt as a soldered control board with pinouts for each module to reduce its size and weight. The board uses a mix of male and female pin headers so that all the on-board modules can be removed and all the external modules can be disconnected from the board. The control board consists of just the microcontroller and the IMU, with the microcontroller’s on-board LED displaying the connection status. The antenna is detached so that it can be optimally positioned, the antenna cable also has grounded cable shielding. The control board has five PWM channels, one for the motor and 4 for servos (we only used 3). The motor channel connects to the ESC which powers the board from the BEC (5v) and supplies power to the servos. The control board circuit consists of the following components: - Waveshare RP2350-Zero (1x) - MPU6050 Inertial Measurement Unit (1x) The antenna is a detached module with the following: - NRF24L01 PA + LNA Antenna Module (1x) - 10nF Ceramic Capacitors (1x) - 100uF Electrolytic Capacitor (1x) The rest of the plane consists of: - 1450Kv Brushless Motor (1x) - 60A ESC (1x) - 2200mAh 4S 14.8V Lithium Battery (1x) - MG90s Servos (3x) The antenna and electromechanical components are adhered or mounted to the plane. |

# Conclusion